# SSH Configuration on a Router Cisco Packet Tracer (Step-by-Step)

## 1. Network Topology

Devices:

- 1 Router

- 1 Switch

- 2 PC

Example IP Addressing:

Router (G0/0): 192.168.1.1

PC (NIC): 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

## 2. Configure Router Interface

Commands:

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

Verify:

show ip interface brief

## 3. Configure PC IP Address

On the PC go to Desktop → IP Configuration

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Test connectivity:

ping 192.168.1.1

## 4. Configure SSH on the Router

hostname R1

ip domain-name netlab.local

username admin secret cisco123

crypto key generate rsa general modu

Choose 1024 bits when prompted.

## 5. Configure VTY Lines for SSH

line vty 0 4

login local

transport input ssh

exit

## 6. Enable SSH Version 2

ip ssh version 2

## 7. Verify SSH Configuration

show ip ssh

show running-config

## 8. Test SSH from PC

Open PC → Desktop → Command Prompt

ssh -l admin 192.168.1.1

Password: cisco123

If successful, you will see:

R1>

## 9. Verify Active SSH Sessions

show ssh

show users

## Steps

When recording your lab for Netlabs Hub, demonstrate:

1. Network topology

2. Router configuration

3. PC configuration

4. SSH login test